Hard Drive DVR System – Playback / Export

of Video Files Directly from DVR

Equipment Required – DVR remote control, DVR keys, USB flash drive (for exporting files if required)

REMOTE CONTROL FUNCTIONS

PLAYBACK OF RECORDED FILES

Turn on vehicle ignition. Set monitor to channel 1 (V1/AV1/CAM1). After a few seconds the “system loading” screen will appear. The DVR is now starting up. After approx. 15-20 seconds, the quad view should appear and show an image from each camera (the 8 channel model will show an 8 screen display with all connected cameras visible).

Point remote control at DVR front panel. Press LOGIN

Select SEARCH. Press ENTER

In the window that opens, any days that have recordings are highlighted in green, days without are in blue.

Use the arrow and number keys to enter the correct date that you wish to view (select each number and use the number keys to overwrite what is currently there).

If required, use the arrow and number keys to enter the time range (if nothing is entered here, all data from the selected date will be loaded).

REC PATH – This selects the storage that you wish to read the data from. In nearly all cases this will be HDD, unless the hard drive has been removed and a backup SD card is present, in which case select SD.

REC TYPE – Here you can select whether you wish to view all video files (GENERAL) or files that have ben created as a result of a particular signal/event the DVR has detected (ALARM). In most cases, any required files will be contained in the GENERAL section.

CHANNEL – if you only wish to view data from one of the installed cameras e.g. nearside, you can select which channel you want and it will filter out the others.

Once you have made your selections, select SEARCH and press ENTER. The DVR will now load all the relevant data.

Use the arrow keys to select the segment you wish to play. Press PLAY. The DVR will load all channels associated with that segment and begin playback (for example, if it is a 4 camera system, all 4 cameras will be shown on playback).

Use the buttons on the DVR remote as shown in the first image to control playback functions. Use RETURN button to go back to the file list at any time. Use the number keys to bring up one of the cameras full screen, and the blue GRID button to return to multi-channel view.

EXPORTING VIDEO FILES TO USB FLASH DRIVE

If you wish to copy recorded files to a USB flash drive to view later, please follow these steps (please note that the DVR will export the original raw files, and if the files are to be viewed by a third party without our PC software installed, the raw files will first need to be converted with the PC software).

Follow the above steps to bring up the list of files for the required date (up to the third image).

Use the arrow and ENTER keys to tick the files you wish to export. Use FIRST/LAST/PREV/NEXT buttons to move between pages of files (there are likely to be multiple pages for each day)

Use the arrow keys to select EXPORT. Press ENTER. Files will begin to copy across to the flash drive (this may take some time depending on the amount of data being exported).

When exporting has completed, turn off DVR by turning the key 90° in the lock. Wait for the lights to go out (apart from the blue which stays on) then remove USB flash drive.

Files may now be played on the PC playback software and converted to a universal format if required. Please see separate guide for instructions for this process.

FOR ASSISTANCE CONTACT TRAFFICANGEL: 01825 768555 / help@trafficangel.co.uk